Find the sequence of observations, i.e., the sequence of RSOs a0, a1, …, aK with observations starting at τ0, τ1, …, τK respectively. The number of observations, K+1, is not fixed in advance. If there are n RSOs in total, then ai∈1, 2, …, n. Let Pj,1, Pj,2 be the timespan during which the jth RSO is in the optical range of the sensor, and let 0, T be the planning horizon (one orbit). The slew time required for positioning the sensor from the jth RSO to the kth RSO, such that the sensor is on the kth RSO at time τ, is defined as tj,kτ.

The observation time on the ith RSO of the sequence is

di=minτi+1-τi-tai-1,aiτi, Pai,2-τi,   i=0, 1, …, K-1

dK=minPaK,2, T-τK-taK-1,aKτK

i.e., an observation ends either when the sensor must start slewing to reach the next RSO in time, or when the RSO leaves the optical range of the sensor, whichever is earlier.

Optimization Problem:

Decision variables: K, a0, …, aK and τ0, …, τK

Maximize: 		i=0Kdi

Subject to:		ai∈1, 2, …, n,   ai+1≠ai

τi∈Pai,1, Pai,2∩0, T

di≥dmin>0

where dmin is the minimum useful observation time. Maximizing idi maximizes the fraction of the orbit, idiT, during which an RSO is in the frame of the sensor. An RSO may be revisited later in the sequence, but not observed twice in succession.